- Návod k použití – CZ      LED svítilna s USB nabíječkou - SW0184RD – LAKAS

- Princip:  Tento produkt pracuje na principu dynama – dobíjecího 3 fázového generátoru a stabilizátoru. Má vestavěné dobíjecí baterie 80 MAH3.6V NI-MH. Je také možné tento přístoj použít jako nabíječku mobilního telefonu, 5,7V.

- Funkce:

- Svítilna s 3 LED diodami s několika módy svícení

- Nabíječka mobilního telefonu pomocí USB konektoru.

- Vlastnosti:

    - dobíjecí generátor

    - vysoká svítivost, maximální jas je 5000mlm

    - už nikdy nepotřebujete baterie

    - ergonomický hezký desing

- Použití:

- Svícení

Odklopte rukojeť, držte přístroj jednou rukou a druhou otáčejte rukojetí rychlostí 2,5-3 otáčky za sekundu (libovolným směrem) po dobu jedné minuty – čímž se nabíjí vestavěná baterie. Zmáčknete-li:

- jednou červený spínač – svítí jedna prostřední LED dioda

      - podruhé – svítí všechna LED diody a)

      - potřetí - svítilna bliká

      - počtvrté - zhasne.

- Nabíjení mobilního telefonu

Odklopte rukojeť. Pod zámkem rukojeti je vstup pro USB kabel, který je součástí balení . Vždy je možné použít libovolný USB kabel pro Váš typ mobilního telefonu. Vložte jeden konec kabelu do vstupu na přístoji a druhý konec do mobilního telefonu. Mobilní telefon poté položte na rovnou stabilní plochu, abyste zabránili pádu.

Držte svítilnu jednou rukou a druhou otáčejte rukojetí rychlostí 2,5-3 otáčky za sekundu. Jakmile se na mobilním telefonu objeví indikátor dobíjení, probíhá nabíjení

- Technické parametry

| Položka | Parametr | Jednotka | Položka | Parametr | Jednotka |
| --- | --- | --- | --- | --- | --- |
| Maximální světelný tok | 5000 | mlm | Frekvence blikání | okolo 2 | HZ |
| Maximální jas | 55000 | Mcd | Doba dobíjení mobilního telefonu po 3 minutách nabíjení | 2-8 | minut |
| Doba svícení – minuta nabíjení (3 LEDs) | >25 | Minute | Životnost USB | 5000 | použití |
| Výstup napětí - naprázdno | 5.7±0.2 | V | Živostnost spínače | 10000 | použití |
| Proud | ≥350 | mA | Životnost LED | 100000 | hodin |
| Maximální výkon | 2.5 | W | Životnost plastovým částí | 5 | let |
| Frekvence blikání | okolo 2 | HZ | Životnost přístroje | 5 | let |

- Upozornění:

- Držte přístroj tak, aby při otáčení rukojetí nedošlo ke zranění prstů obou rukou.

- Nesviťte přímo do očí, světlo může způsobit poškození zraku.

- Pro zachování maximální životnosti přístroje doržujete následující pokyny:

    - neotáčejte rukojetí déle než 5 minut, pokud je baterie nabitá a intenzita svícení je normální a méně než 15 minut pokud je přístroj vybitý.

    - Pokud je světlo málo jasné, znamená to, že je baterie vybitá. Prosím vypněte svítilnu nebo chvíli dobíjejte rukojetí.

- Zásady nabíjení:

    - ideální rychlost otáčení je 2,5-3 otočky za vteřinu

    - neotáčejte příliš rychle, násilně nebo prudce

    - nikdy neměňte náhle směr otáčení

    - neotočejte příliš pomalu – můžete tím baterii poškodit

    - při nabíjení mobilního telefonu otáčejte nepřetržitě, aby nedocházelo ke kolísání napětí, což může být negativní vliv na dobíjení

- Vhodné použití pro nouzové situace – nemusíte otáčet příliš dlouho pro základní použití.

- Pro prodloužení životnosti je vhodné přístroj jednou za měsíc po dobu jedné minuty dobíjet.

- Nepoužívejte v prostřetí s vysokou vlhkostí, vysokou nebo příliš nízkou teplotou. Nevystavujte žáru ani mrazu.

- Určeno pro nabíjení mobilního telefonu při stavech nouze nebo na cestách, kde není k dispozici originální nabíječka.

- Je možné, že přístroj se bude zahřívat při dlouhodobém otáčení rukojetí.

- Frekvence blikání se může měnit, záleží na vlivu prostředí, teploty, stavu nabití baterie a rychlosti otáčení.

- Pokud bude svítilna během nabíjení vypnutá, urychlíte proces nabíjení.

- Velikost přístroje:120*53*32 mm

Váha:108 g

User Manual EN - SW0184RD     LED Flashlight With USB Charger

- Working Principle:

This product adopts the principle of cranking for power by 3-brushless generator,

which after commutating, voltage

stabilizing and filtering to charge its built-in

80 MAH3.6V NI-MH chargeable battery. It can also charge a mobile

phone through

wire and connector by the output 5.7V voltage.

- Functions :

1. Cranking to generate power which is used for lighting of 3 φ5LEDs .

2. Cranking for power to charge mobile phone through wire and connector .

- Features :

- Cranking for power generation.

- High luminance: The Max. brightness is 5000 mlm.

- Convenient to use, never need batteries.

- Exquisite sculpt, lovely, convenient to carry .

- Main Technical Parameters :

| Item | Parameter | Unit |
| --- | --- | --- |
| The Max. Luminous flux | 5000 | mlm |
| The Max. brightness | 55000 | Mcd |
| Lighting time after being charged for 1 minute (3 LEDs) | >25 | Minute |
| Voltage output (no-load) | 5.7±0.2 | V |
| 10 Ωoutput current (load) | ≥350 | mA |
| The Max. power output | 2.5 | W |
| Flash frequency | About 2 | HZ |
| Mobile phone’s working time after being charged for 3 minutes | 2-8 | Minute |
| USB’s lifespan | 5000 | Time |
| Switch lifespan | 10000 | Time |
| LED lifespan | 100000 | Hour |
| The plastic part lifespan | 5 | Year |
| The whole unit lifespan | 5 | Year |

- Operation Method:

- Lighting:

①　Unlock the handle, hold the unit and crank the handle by the other hand at the speed of 2.5~3 circles per second for about one minute to charge for its built-in chargeable battery.

②　The switch model: Switch on once , the LED in middle lights up; Switch on again, 3　LEDs are on; Switch on for the third time : 3 LEDs flash; Switch on once more, the 3 LEDs are off.

- Charge for mobile phone:

Unlock the handle, insert one end of the charging cables into USB electrical outlet and the other end to the mobilephone; Hold the unit and crank the handle by the other hand at the speed of 2.5~3 circles per second, when the mobilephone gives indication, it means charging.

- Cauting:

1. Pay attention to the holding gesture when cranking so as not to hurt your fingers.

2. Don’t point at eyes directly so as not to hurt the eyes.

3. In order to assure the using life of the battery and mechanical parts,pls pay attention to the things below:

a) Do not crank it for more than 5 minutes when the power of the battery is enough (lighting normally) and less than 15 minutes when it runs out of power.

b) When the light is dim,it means the battery is lack of power badly, pls turn off the light or crank it for a while.

c) Cranking it as even as possible (keep rotating steadily, 2.5 or 3 circles per second is best). Avoid cranking too violent and quick or changing the cranking direction abruptly. Too slowly cranking would make current weak and influence the charging effeciency, and too violent cranking possibly damages the driving system. Keep cranking to charge a mobilephone continuously, otherwise the power consumption will increase and affect the charging effect.

d) This product can be used to meet an emergency , so do not crank it for a long time or too violent.

e) In order to prolong the lifespan of the battery, you should crank it for at least one minute once a month.

4. Do not place this product in where temperature or humidity is high . Do not drop or hit this product.

5. This product can only charge mobile phone to meet an emergency, so do not charge mobile phone for a long time.

6. The product will be a little hot if cranking for a long time .It is normal that the flashing frequency be a little different as affected by environment,temperature and the cranking speed.

8. Turn off the light when charging it in order to enhance the efficiency of charging .

- Size:120*53*32 mm

Weight:108 g